CITY OF LOS ALTOS

##### Community Development

Building Division

Submittal Hours:  8:00 a.m. – 11:30 a.m. & 1:00 p.m. – 4:30 p.m.

Telephone No. (650) 947-2752 / Inspection Line (650) 947-2754

SUBJECT:  BATH REMODEL/UPGRADES

WATER CONSERVING PLUMBING FIXTURES REQUIRED THROUGHOUT RESIDENCE

Three sets of plans shall be submitted for review and approval for code compliance by the City Building Inspector. One set is retained for city records, one set is sent to the Santa Clara County Assessor, and one set is returned to the applicant to be used for construction and shall be made available at each and every inspection requested.

- Plans do not have to be prepared by a professional, but shall be drawn to a professional type quality at a ¼” per foot scale, showing floor plan, fixtures, wall legend, and include notes meeting current applicable codes.

(Note: During plan check the plans examiner may determine that structural

changes require calculations from a licensed architect or engineer.)

- Floor plan wall legend shall show existing, proposed, and removed walls. If wall changes are extensive provide separate plans for existing/proposed for clarity. If fixtures are going to be relocated include minimum dimensions for the following:

  - Toilets require 15” to centerline of drain to any vertical surfaces on each side, 30” total. Toilets front clearance is 24” with the exception of a door swing into the required clearance.

  - Newly built shower enclosures are required to have a minimum area of 1024 square inches and shall also be able to accommodate a 30” diameter circle (elbow room). Indicate that glass enclosures shall be tempered and doors shall have a clear opening of 22”.  Shower/tub doors shall be either sliding or outward swing configured.  Exterior windows located in existing or new shower /tub locations shall be noted as tempered safety glass. Note shower valve shall be pressure/balance type.

  - Age friendly curbless shower designs shall show structural cross section indicating how floor framing will be altered to accommodate minimum 2” dropped threshold.  Show 2% slope to trench drain or (2) standard split drains at back of shower enclosure, and provide note indicating floor waterproofing will extend a minimum of 60” out from shower entrance into bathroom floor area.

3.) Mechanical/Electrical requirements shall be noted on plans:

  - Mechanical ventilation (bath fan) shall be provided. GFCI

protected is required if located within tub/shower enclosure.

- Sinks shall be provided with a GFCI tamper resistant outlet within 36” and shall be wall mounted. Concealed outlets in cabinets will not be counted as a required outlet. Newly created bathrooms will require a dedicated 20 amp circuit originating from the house circuit breaker panel.

- California Energy Code (T-24, Part 6) dictates all lighting efficiency for homes. Bathrooms are required to have either high efficacy LED/ fluorescent throughout or have all incandescent or low voltage lighting controlled by switching that is the “Manual ON/Occupancy Sensor” type.  As of July 1, 2014 all lighting shall be “Vacancy Switch Controlled”.

- If the lighting choice is recessed cans, indicated on plans I.C./A.T. (Insulation Cover/ Air Tight) .

- Indicate bath tub as soaking or whirlpool type. Whirlpool type tubs

will require at least one dedicated 20 amp originating from the

circuit breaker panel. Some manufactures require two circuits.

Provide listing and manufacture’s installation instructions at time of

inspection

F.)  Steam showers shall include manufacturer’s specs for steam

generator.  Show access for generator and plan note to slope

ceiling.

4.) Demolition of existing bath is permitted only after plans have been

reviewed, approved, and permit fees have been paid. Permit fees are

based on the total  valuation of the work being done as indicated on the

application.  Permit application, fee schedules, allowed construction hours,

and permitted dumpster debris box information are available thru the

Building Division at Los Altos City Hall.

5.) Inspections can typically be arranged within one or two days advanced

notice and are required before concealment of any framing, electrical,

plumbing, ducting changes. The combination of all above mentioned

trades shall be done at the same time. Additional inspections are required

for new shower pans, sheetrock, waterproofing and tile backing.  			   Requests for inspections prior to the work being ready may result

in a re-inspection fee due prior to the next inspection request and

may delay the inspection scheduling process. Final inspection is required

by the City of Los Altos and per the California Building Code.

WATER CONSERVING PLUMBING FIXTURES   	REQUIRED THROUGHOUT RESIDENCE

Effective January 1, 2014, State of California Senate Bill 407 requires non-compliant plumbing fixtures to be replaced with water conserving plumbing fixtures when a property is undergoing an alteration, improvements, or addition. Verification is required prior to final permit approval by certification from the property owner. Repairs or maintenance will not trigger this requirement. This law applies only to properties built on or before January 1, 1994.

Max Flow Rates :

Water closets…….1.28 gpf (gallons per flush)

Shower heads…….2.00 gpm

Lavatory faucets…1.50 gpm

Kitchen faucets…..1.80 gpm

Note: The effective flush volume of dual flush toilets is the average flush volume of two reduced flushes and one full flush, 3 flushes total.

Standard : 1.28 x 3 flushes = 3.84 gallons.

Dual flush: 1.00 + 1.00 + 1.6 = 3.60 < allowed 3.84 OK.

References : California Civil Code Sec. 1101.1-1101.8 and

2013 Cal-Green Sec. 301/4.303.1